VISIT SHEET

| COMPANY | Name: Massimo Dutti India Private Limited Date: 10/07/2026 |  |
| --- | --- | --- |
| Project/Site Name | Massimo Dutti India Private Limited |  |
| Technical description of the report |  | General Work, scaffold work, electrical work, welding work. |  |
| Observation |  | Subcontratas en obra / Subcontractors on site: - |  |

| Control Measures/ (Identify the applicable risk and conduct a preventive analysis.) |  |  |
| --- | --- | --- |
| 1. General worksite conditions Estado general de obra | 1. Poor housekeeping observed at multiple locations, with helmets, bags, buckets and debris left on pedestrian walkways. |
| 2. Risk of falls from heights Riesgos por caídas a distinto nivel | 1. Toe boards were missing on most mobile scaffolds in use. 2. Wheel locks of the mobile scaffold were found unlocked during use. 3. Mobile scaffold was used without a safe means of internal access (ladder not provided). 4. Worker was standing on a cement brick placed on the scaffold platform to increase working height. |
| 3. Risk of same level falls Riesgo por caídas al mismo nivel | 1. Electrical cables were routed directly on the floor creating slip/trip hazards. |
| 4. Electrical risk Riesgo eléctrico | 1. Electrical cable found hanging loosely across a pedestrian walkway, creating a trip hazard. 2. Electrical cables were routed directly on the floor creating slip/trip hazards. |
| 5. Risk of falling objects Riesgo por caída de objetos | 1. No warning signs was provided below the scaffold during overhead work. |
| 6. Excavations: risk of collapse Riesgo por desplome/derrumbe | NA |
| 7. Risk of injury from objects/tools Riesgo de corte, golpe con objetos, herramientas | 1. Worker on the scaffold was observed without wearing a safety helmet. |
| 8. Risk of being run over by vehicles or machinery and/or over | NA |

VISIT SHEET

Control Measures

1. Ensure all workers are trained, competent, and equipped with the required PPE. 2. Verify that chemicals are properly labeled, stored, and handled according to safety requirements. 3. Ensure machinery is regularly inspected, maintained, and operated by authorized personnel only. 4. Maintain up-to-date records for workforce compliance, chemical management, and machinery inspections. 5. Identify potential risks and implement preventive actions to eliminate or minimize hazards before incidents occur.

| Non-compliance |  |
| --- | --- |
| Workers | NA |
| Machinery | NA |
| Chemicals | NA |

| Foreman | DNI / ID no. / Passport no. | Signature |
| --- | --- | --- |
| First and family name |  |  |

| Auditor | Signature |
| --- | --- |
| First and family name Pushpender Sharma | Pushpender Sharma |